PRACTICAL 1: Write a program to generate a discrete time unit sample, unit step, ramp and sinusoidal signal. SCILAB CODE: (a) Unit step signal

clf(); clc(); clear();

L=10 n=-L:L

x=[zeros(1,L),ones(1,L+1)] // Plot the signal plot2d3(n, x, style=6); xlabel('Time');

ylabel('u[n]');

title('Discrete Unit Step Sequence');

// for bubbles plot2d(n, x, -3); // Axis settings a = gca();

a.data_bounds = [-10 0; 10 2];

a.x_location = "origin";

a.y_location = "origin";

(b) Unit Impulse Sequence

clf(); clc(); clear(); n= -10:10; x= zeros(1, length(n)); x(n == 0) = 1;

plot2d3(n,x, style=5); a = gca();

a.data_bounds = [-5 0; 5 2];

a.x_location = "origin";

a.y_location = "origin"; title("Discrete-Time Unit Impulse"); xlabel("n")

ylabel("x") xgrid();

// for bubbles plot2d(n, x, -3);

(c) Ramp Signal clf();clc();clear();

t = 0:1:15; // define the signal ramp = t .* (t >= 0);plot2d3(t, ramp, style=5); title(" Discrete Ramp Signal"); xlabel("t"); ylabel("r(t)"); a = gca(); plot2d(t, ramp, -3);

a.data_bounds = [0 0; 10 10.5];

a.background = color("grey");

(d) sinusoidal signal clf(); clc(); clear(); t = -0:0.1:20; // Define the signals x1 = sin(t); // Ploting of sint plot2d3(t,x1, style=5);

title('sin(x)'); ylabel('Amplitude'); xlabel('Time'); plot2d(t, x1, -3);

PRACTICAL 2 : write a program to find the Fourier transform of a sequence. SCILAB CODE :

clc;clear;clf;

// Define sequence n = 0:5; x = ones(1, length(n));// unit step // Plot 1:

subplot(3,1,1);

for i = 1:length(n)

plot([n(i) n(i)], [0 x(i)], 'b-');

# end

plot2d(n, x, -3);title("Unit Step Sequence x[n]"); xlabel("n"); ylabel("x[n]"); xgrid();

// Frequency range w = -%pi:0.01:%pi; // DTFT

X = zeros(1, length(w));

for k = 1:length(w)

X(k) = sum(x .* exp(-%i * w(k) * n)); end

// Plot 2: Continuous DTFT subplot(3,1,2); plot(w, abs(X), 'b-'); title("Magnitude of DTFT |X(w)|"); xlabel("Frequency (rad/sample)");

ylabel("|X(w)|");xgrid(); // Plot 3: Discrete DTFT subplot(3,1,3); w_d = -%pi:%pi/10:%pi; X_d = zeros(1, length(w_d)); for k = 1:length(w_d)

X_d(k) = sum(x .* exp(-%i * w_d(k) * n)); end for i = 1:length(w_d)

plot([w_d(i) w_d(i)], [0 abs(X_d(i))], 'r-'); end

plot(w_d, abs(X_d), 'ro'); title("Discrete Magnitude |X(w)|"); xlabel("Frequency (rad/sample)");

ylabel("|X(w)|"); xgrid();

PRACTICAL 3 : Write a program to find the pole zero plot of a function. SCILAB CODE :

clc;clear;clf;

// EXAMPLE :- H(Z)= 1+0.5Z^-1 / 1-0.8Z^-1 + 0.2Z^-2. // Zeros and poles z = [0 -0.5];

p = [0.4+0.2*%i 0.4-0.2*%i];

// Plot zeros plot(real(z), imag(z), 'bo'); xgrid();

// Plot poles

plot(real(p), imag(p), 'rx'); // Plot unit circle t = 0:0.01:2*%pi;

plot(cos(t), sin(t), 'k--'); xlabel("Real Axis"); ylabel("Imaginary Axis"); title("Pole-Zero Plot"); legend("Zeros","Poles","Unit Circle");

PRACTICAL 4 : Write a program to find the functions Z transform and inverse Z transform

SCILAB CODE :

clc;clear; // define n n = 1:4;

// original sequence x = (0.5).^n;

// define new variable w = z^-1 w = poly(0, 'w');

// -------- Z-TRANSFORM -------X = 0;

for k = 1:length(n)

X = X + x(k) * w^(n(k)); end

disp("Z-Transform X(w):");

disp(X);

// -------- INVERSE Z-TRANSFORM -------N = length(n); x_inv = zeros(1, N); for k = 0:N-1 x_inv(k+1) = coeff(X, k); end

disp("Inverse Z-Transform x[n]:");

disp(x_inv);

OUTPUT:

PRACTICAL 5 : Write a program to find the circular convolution of 2 sequences.

SCILAB CODE :

clc;clear;

// Input sequences x = [2 -1 3 0 1 4]; h = [1 2 -2 1 0 3];

N = length(x); y = zeros(1, N); // Circular convolution for n = 1:N

for k = 1:N

index = pmodulo(n - k, N); // always 0 to N-1 y(n) = y(n) + x(k) * h(index + 1); // shift to 1-based end

# end

disp("Result y(n):");

disp(y);

OUTPUT:-

PRACTICAL 6 : Write a program to find the DFT of a sequence using direct method. SCILAB CODE :

clf(); clc(); clear();

// input sequence x = [1.5 -2.3 0.7 4.1];

// length

N = length(x);

// initialize

X = zeros(1, N); // DFT (Direct Method) for k = 0:N-1

# for n = 0:N-1

X(k+1) = X(k+1) + x(n+1)*exp(-%i*2*%pi*k*n/N); end end

// Magnitude mag = abs(X); // Phase phase = atan(imag(X), real(X)); phase(abs(phase) < 1e-10) = 0;

// display disp("DFT:"); disp(X); disp("Magnitude:"); disp(mag); disp("Phase (radians):"); disp(phase);

PRACTICAL 7 : Write a program to find the DFT of a Sequence using FFT. SCILAB CODE :

clc(); clear();

// input x = [1 2 3 4];

N = length(x);

// FFT

X = fft(x, -1);

// magnitude & phase mag = abs(X); phase = atan(imag(X), real(X));

phase(abs(phase) < 1e-10) = 0;

disp("DFT:"); disp(X); disp("Magnitude:"); disp(mag); disp("Phase (radians):"); disp(phase);