1- Aim: Write a program to generate discrete time Unit Sample, Unit Step, Unit Ramp and Sinusoidal sequences.

Scilab Code:

clc; clear;

n = -5:5;

// Unit Sample – convert boolean to double

x = double(n == 0);

subplot(2,2,1);

plot2d3(n, x);

title("Unit Sample Sequence");

xlabel("n"); ylabel("x[n]");

// Unit Step

u = double(n >= 0);

subplot(2,2,2);

plot2d3(n, u);

title("Unit Step Sequence");

xlabel("n"); ylabel("u[n]");

// Unit Ramp

r = n .* double(n >= 0);

subplot(2,2,3);

plot2d3(n, r);

title("Unit Ramp Sequence");

xlabel("n"); ylabel("r[n]");

// Sinusoidal

n2 = 0:20;

f0 = 0.1;

s = sin(2 * %pi * f0 * n2);

subplot(2,2,4);

plot2d3(n2, s);

title("Sinusoidal Sequence");

xlabel("n"); ylabel("x[n]");

2- Aim: Write a program to find the Fourier Transform of a sequence.

Scilab Code:

clc;clear;clf;

// Define sequence

n = 0:5;

x = ones(1, length(n));// unit step

// Plot 1:

subplot(3,1,1); for i = 1:length(n)

plot([n(i) n(i)], [0 x(i)], 'b-'); end

plot2d(n, x, -3);title("Unit Step Sequence x[n]"); xlabel("n");

ylabel("x[n]"); xgrid();

// Frequency range

w = -%pi:0.01:%pi;

// DTFT

X = zeros(1, length(w));

for k = 1:length(w)

X(k) = sum(x .* exp(-%i * w(k) * n)); end

// Plot 2: Continuous DTFT

subplot(3,1,2); plot(w, abs(X), 'b-');

title("Magnitude of DTFT |X(w)|"); xlabel("Frequency (rad/sample)"); ylabel("|X(w)|");xgrid();

// Plot 3: Discrete DTFT

subplot(3,1,3);

w_d = -%pi:%pi/10:%pi;

X_d = zeros(1, length(w_d)); for k = 1:length(w_d)

X_d(k) = sum(x .* exp(-%i * w_d(k) * n)); end

for i = 1:length(w_d)

plot([w_d(i) w_d(i)], [0 abs(X_d(i))], 'r-'); end

plot(w_d, abs(X_d), 'ro'); title("Discrete Magnitude |X(w)|"); xlabel("Frequency (rad/sample)"); ylabel("|X(w)|");

xgrid();

3- Aim: Write a program to find the pole-zero plot of a function.

Scilab Code:

clc; clear; clf;

// Unit circle points (used in all 3 subplots)

t = linspace(0, 2*%pi, 300);

uc_x = cos(t);

uc_y = sin(t);

// CASE 1: num < den  (more poles than zeros)

// H(z) = 1 / (z^2 - 0.5z + 0.5)

num1 = [1];              // degree 0  --> 0 finite zeros

den1 = [1, -0.5, 0.5];  // degree 2  --> 2 poles

z1 = roots(num1);   // zeros

p1 = roots(den1);   // poles

subplot(1,3,1);

plot(uc_x, uc_y, 'b--');          // unit circle

a = gca(); a.auto_clear = 'off';

if length(z1) > 0 then

plot(real(z1), imag(z1), 'go', 'MarkerSize', 8);

end

plot(real(p1), imag(p1), 'rx', 'MarkerSize', 10);

a.auto_clear = 'on';

xgrid();

title("Case 1: num < den");

xlabel("Real Axis"); ylabel("Imaginary Axis");

legend(["Unit Circle","Zeros","Poles"]);

// CASE 2: num = den  (equal number of poles and zeros)

// H(z) = (z - 1) / (z - 0.5)

num2 = [1, -1];       // degree 1  --> 1 zero

den2 = [1, -0.5];     // degree 1  --> 1 pole

z2 = roots(num2);

p2 = roots(den2);

subplot(1,3,2);

plot(uc_x, uc_y, 'b--');

a = gca(); a.auto_clear = 'off';

plot(real(z2), imag(z2), 'go', 'MarkerSize', 8);

plot(real(p2), imag(p2), 'rx', 'MarkerSize', 10);

a.auto_clear = 'on';

xgrid();

title("Case 2: num = den");

xlabel("Real Axis"); ylabel("Imaginary Axis");

legend(["Unit Circle","Zeros","Poles"]);

// CASE 3: num > den  (more zeros than poles)

// H(z) = (z^2 - 1.5z + 1) / (z - 0.5)

num3 = [1, -1.5, 1];   // degree 2  --> 2 zeros

den3 = [1, -0.5];      // degree 1  --> 1 pole

z3 = roots(num3);

p3 = roots(den3);

subplot(1,3,3);

plot(uc_x, uc_y, 'b--');

a = gca(); a.auto_clear = 'off';

plot(real(z3), imag(z3), 'go', 'MarkerSize', 8);

plot(real(p3), imag(p3), 'rx', 'MarkerSize', 10);

a.auto_clear = 'on';

xgrid();

title("Case 3: num > den");

xlabel("Real Axis"); ylabel("Imaginary Axis");

legend(["Unit Circle","Zeros","Poles"]);

4- Aim: Write a program to find a function's Z transform and inverse Z transform.

Scilab Code:

clc; clear;

a = 0.5;

N = 20;

n = 0:N-1;

x = a .^ n;

M = 512;

// Zero-pad x to M points BEFORE calling fft

x_pad = [x, zeros(1, M - N)];

// Forward DFT (Z-transform on unit circle)

X = fft(x_pad, -1);

mag = abs(X);

f = (0:M-1) / M;

// Inverse DFT: fft(X, 1) in Scilab is the normalised IDFT

x_rec = real(fft(X, 1));

subplot(3,1,1);

plot2d3(n, x);

title("Original Sequence x[n] = (0.5)^n u[n]");

xlabel("n"); ylabel("x[n]");

subplot(3,1,2);

plot(f(1:M/2), mag(1:M/2));

title("Z-Transform Magnitude |X(e^jw)|");

xlabel("Normalised Frequency"); ylabel("|X|");

subplot(3,1,3);

plot2d3(0:N-1, x_rec(1:N));

title("Recovered via Inverse ZT");

xlabel("n"); ylabel("x_rec[n]");

5-Aim: Write a program to find the circular convolution of two sequences.

Scilab Code:

clc; clear;

x1 = [1, 2, 3, 4];

x2 = [1, 1, 1, 1];

N  = max(length(x1), length(x2));

// Zero-pad both to length N

x1 = [x1, zeros(1, N - length(x1))];

x2 = [x2, zeros(1, N - length(x2))];

// Circular convolution = IDFT( DFT(x1) * DFT(x2) )

X1 = fft(x1, -1);

X2 = fft(x2, -1);

Y  = X1 .* X2;

y  = real(fft(Y, 1));   // fft(.,1) is normalised IDFT in Scilab

disp("x1 = "); disp(x1);

disp("x2 = "); disp(x2);

disp("Circular Convolution y = "); disp(y);

6-Aim: Write a program to find the DFT of a sequence using the direct method.

Scilab Code:

clf(); clc(); clear();

// input sequence

x = [1.5 -2.3 0.7 4.1];

// length

N = length(x);

// initialize

X = zeros(1, N);

// DFT (Direct Method)

for k = 0:N-1 for n = 0:N-1

X(k+1) = X(k+1) + x(n+1)*exp(-%i*2*%pi*k*n/N); end

end

// Magnitude

mag = abs(X);

// Phase

phase = atan(imag(X), real(X)); phase(abs(phase) < 1e-10) = 0;

// display disp("DFT:"); disp(X); disp("Magnitude:"); disp(mag);

disp("Phase (radians):"); disp(phase);

7-Aim: Write a program to find the DFT of a sequence using FFT.

Scilab Code:

clc(); clear();

// input

x = [1 2 3 4];

N = length(x);

// FFT

X = fft(x, -1);

// magnitude & phase

mag = abs(X);

phase = atan(imag(X), real(X));

phase(abs(phase) < 1e-10) = 0;

disp("DFT:");

disp(X);

disp("Magnitude:");

disp(mag);

disp("Phase (radians):");

disp(phase);